Modifying & Programming

The Philips / Simoco PRP80

Michel G Mitaux

Version 3.8 September 2004

email:  mitaux8030@hotmail.com

Introduction:

The Philips / Simoco / ComGroup PRP 80 series of handheld radios complement the PRM8030 mobile units.  The PRP80 is a unique radio as it appears that it was a hybrid design using the Shinwa SH405 handheld RF stages, and a custom Philips designed control board.  As such, the PRP80 has features and functions very close to the PRM8030 series of mobiles, with a few features from the 8040 thrown in too.  Little information is known about the PRP80, at least in hobbyist circles, and so much information given here has been learnt from scratch, and there may be incorrect assumptions made or other errors, in which case if you do have something to add or correct, I would be most pleased to hear about them and have them corrected.

Getting to know the PRP80:

PRP80s as far as I know came in only three versions: PMR only, dual mode, and trunking only.  In this document, I will only discuss the PMR side of things, since that is the only aspect that us hobbyists can really make use of.  Of course this applies to the dual mode units when in PMR mode, too.

The PRP80 has features such as CTCSS, DTMF and selcall built in as standard, scanning with priority and user selectable channels, five scanning banks, voting & multiax, a received signal strength indicator, high and low power selection nominally 5 and 1 watts respectively, TOT, emergency & alarm features, and 100 channel memories although this can be expanded on using the FPP 'Xtra' version.  All these features operate in pretty much the same way as they do on the PRM8030, and there are some new features, too, such as being able to store 15 frequently dialed DTMF and 15 frequently called selcall numbers.

The PRP80 also has alphanumeric text labels that you can associate to each channel, voting group and multiax set.

The original Shinwa unit had DTMF as an option, which was installed as standard on the PRP80, and there was a scramble option, too, which was not available with the PRP80.

There are four function buttons available, and if you assign one of them as the 'shift' key, you can get an extra 12 functions via the front keypad, just as you can with the 'keypad mode select' function of the PRM8030.  Not quite all of the functions available on the PRM8030 are available on the PRP80, but all the important ones are there, such as scan, power hi / low, squelch defeat, CTCSS defeat, repeater defeat, selcall reset, etc, and some functions are permanently available through the PRP80 firmware, such as scan add/delete and keypad mode select.  The main feature missing is a variable squelch / mute adjustment, this being a fixed internal setting, although there is a user selectable feature to disable the squelch completely.  As you read through this, you will find that many functions, features and concepts of the PRM80 have been carried through to the PRP80, and because of this, you might like to read my PRM80 modification document first, since many of the modifications detailed there, especially those relating to software modifications, work equally as well in concept (but not exact execution) on the PRP80, too.

The PRP80 comes in the following bands:

B0: 132 - 156 MHz

A9: 146 - 174 MHz

TM: 400 - 440 MHz

U0:  440 - 470 MHz

W1: 470 - 500 MHz

W4: 500 - 520 MHz

Note the lack of an E band (70 to 85 MHz).  Also, some versions of FPP show the following:

B0 band as being 132 to 156 MHz, but allow only 136 to 150 MHz to be programmed.

A9 band as being 146 to 174 MHz, but allow only 150 to 174 MHz to be programmed.

TM band as being 400 to 430 MHz, but allow only 403 to 430 MHz to be programmed.

W4 band as being 480 to 520 MHz, but allow only 494 to 512 MHz to be programmed.

(I think this list is most accurate and representative for the PRP80s)

Also note that FPP versions 4.0, 4 Xtra and 4.1 are the only versions available that can program the PRP80.

I am told that the B0 units were never made, and that U0 & W1 units were rare, most were made for A9 and TM bands, with a few in W4 band.

A look under the hood:

To open the PRP80 up, remove the speaker mike cover (if any) and remove the two screws near the speaker mike connector.  You will also need to take off the battery and remove the metal battery guide frame being held on by two screws.  When removing this frame, be careful of the two square shaped rubber grommets that come out with the frame - don't loose them.  Next, the whole plastic outer case complete with keypad buttons will slide down with some gentle pressure - or if you like, push the 'guts' of the radio up out of the case using the battery contact 'block'.  Once the case begins to slide, separate the top curved part of the 'hump back' so that it stays with the top panel of the radio, don't let it come away with the case.

Now that you have the radio 'guts' out, lay the radio down so the keypad & display face up.  Look for the screws that hold the keypad & display to the frame of the radio via some 'tabs' on the frame.  If you remove only those four screws, the whole front panel consisting of the keypad, display, speaker and mike, will hinge over to the right - be careful, the 'hinge' is only a few flexitracks, which can be damaged easily.  Below this panel are the RF stages of which pretty much most of the adjustments are located.  There are the four helical filters, a mute setting, a 'tone' (CTCSS & selcall) deviation adjustment, and overall deviation adjustment, a PLL deviation phase null adjustment (marked REF), a frequency netting adjustment, a high & low power adjustment, and several IF stage adjustments.  Below a copper shim shield stuck to a few modules is - another adhesive foil cover! - which covers the two VCO slug adjustments.  Look at photo prp80uhf.jpg for explanation of the adjustments.

Looking at the rear of the radio, below the first metal shield is the CTCSS / selcall / DTMF board, and below that is the control board.  There is a single adjustment located here that adjusts the selcall level alone.   That is pretty much it.

Software mods:

Programming the PRP80 with channels, features etc. is accomplished by using a PC with software called FPP (Field Personality Programmer) and an interface between the PC and the PRP80's speaker mike connector.

To enable extra channels (theoretically up to 200 of them) use the modified version of FPP called FPP 4 Xtra.  There are some additional problems using the extra channels, with things like not being able to name voting or multiax groups placed in channel numbers above 99, inability to scan channels or manually tag them for scanning above channel 99, and some other weird occurrences, so you will just have to fiddle with it and see how it works and make changes on the fly until things work as they should if using more than 100 channels.

To enable the selcall decode & display feature, you do exactly the same thing as you would on the PRM8030 - program in a group identity of 1VVVV.  Again, just like the PRM8030, trying to use all V's does not work.  Refer to the prm80mod.doc for further information.

Free entry of selcall digits in the PRP80 is achieved by pressing the 'menu' button until 'selcall' is displayed, and then turn the top selector past the '15' position to the star, and then simply dial your number and press enter.  This method can not, however, transmit a gap between the lead in tone and the selcall string, just like on the PRM8030.  While most UHF CBs can cope with the lack of this gap, another PRM80 or PRP80 can not handle this if the lead in tone and the first number of the selcall string is the same (i.e. your lead in tone is set to 1, and the first digit of the selcall is also a 1, which is a pretty common scenario on UHF CB).  A way around this is to do the following: in your 15 pre-programmed identities, program up a memory as F1VVVV, another as F2VVVV, F3VVVV and so on to F0VVVV.  Then in your status number & naming screen, add a single (or more if you wish) status number and name.  Once programmed, you will be able to enter from the keypad of the PRP80 up to four digits into the status (i.e. into VVVV) and then by selecting one of the pre-programmed selcall identities, you can precede your four digit 'status' entry by a digit that you choose in the 'selcall' entry, making up your five digit selcall string that you send out.  This method adds in the gap between the lead in tone and the selcall string.  Convoluted, yes, but it does work.  I guess you could program up your selcall entries as something like F1VVVV12345, so that when you do call someone, 12345 shows on their radio as who the call came from (this relates to the Australian UHF CB service only)

(Authors note: this will void the type approval of the radio, and as such will not be legal to use.  Proceed at your own risk…)

Out of band frequencies can be entered using exactly the same method as you do for the PRM8030 series - again refer to the prm80mod.doc for information on how this is done.  Be aware that the VCO has its limits, and if you try to enter a low frequency of say 430 MHz and an upper frequency of 480 MHz, the VCO simply will not cope with this spread.  The coverage of PRP80 units seems to be about 30 MHz for each band, and this entire band can be shifted via tuning of the VCOs up or down anywhere between 10 and 20 MHz total; for example if you had a W1 band unit, normally 470 to 500 MHz, it may be able to do 450 to 480 MHz or 490 to 520 MHz and anywhere in between, but there will be the normal manufacturing tolerances and individual differences that will cause you to achieve different results.  The five W4 band units I have so far played with will quite happily tune down to UHF CB, but will not quite make it to the 468 MHz police allocations; sure, the frequencies can be programmed in and accepted, but 470.100 MHz was the lowest I could reliably achieve PLL lock.  There is a fix to this, however, by replacing the RX VCO metal tuning slug with a ferrite one, the PLL will lock down to about 465 MHz.

Hardware mods:

(Authors note: this will void the type approval of the radio, and as such will not be legal to use.  Proceed at your own risk…)

Just like the PRM80 series, you can swap RF boards over in the radios to turn a VHF unit into a UHF one or vice versa.  Because the keypad and display and top panel and frame normally form the one unit along with the RF board, it is easier to think along the lines of swapping the control boards onto the RF assembly to complete the unit.  Swapping the control boards on to a RF assembly that you want is pretty easy - simply detach the control boards from your 'old' unit by undoing all screws that hold the selcall / CTCSS board on and carefully using your fingernail to pull out the retaining collar that holds the flexitrack and remove the flexitrack to allow the board to come free.  Do the same to the pair of flexitracks for the microprocessor board after removing its shield, and you are ready to transplant these to the other 'new' RF assembly.  Don't forget to use FPP to modify the hardware code to suit the new band; see the prm80mod.doc for more info on doing this - and if you have trouble doing so, use FPP V4.0 to change the hardware code.  (I found V4.1 is a little flaky when changing hardware codes - PC lockup being the problem).

Of course, the same comments relating to the PRM80 relate to the PRP80 - if you have a VHF unit, and want to turn it into a UHF one, you need to swap the RF hardware, and reprogram the radio as a UHF unit.  Just reprogramming the unit alone will not magically change a VHF unit to UHF.  This PC programming isn't black magic, you know.  Moving right along...

As mentioned above, the PRP80 has a pair of tunable VCOs, one for RX, one for TX.  Within reason, these may be tuned up or down (10 or maybe 20 MHz if you are lucky) to partially cover an adjacent band.  To do this you will need to remove the copper shim that covers the VCO top can - it is both soldered on and uses an adhesive to fix it in place, so use some desoldering wick to remove as much solder as possible before very gently prying up the copper shim.  Underneath this you will find another adhesive foil sticker covering the VCO tuning holes - again very carefully remove this sticker to gain access.  Tuning the VCOs is easy - adjust the TX VCO for stable transmit at the lowest and highest frequencies, while transmitting into a dummy load, and powering the radio from a decent (2 amp or more) regulated 8 volt power supply, or select low power on the radio and you should be able to use a low current regulated 8 volt supply.  When tuning, try to use a non-metallic item, but given the tiny size of the VCO slugs, you may have to use a tiny metal screwdriver - just be sure to only check your adjustments after you have removed the screwdriver, so the metal does not affect the VCO while you try it out.  Try to get the VCO as 'centered' as possible to meet your coverage requirements - this is to provide some margin so that heat (from transmitting or being left in a car interior) or cold that may affect the VCO does not end up putting you 'off air' until the radio returns to room temperature.  RX VCO tuning is not hard either - look for a strong and stable Local Oscillator (LO) signal 45 MHz above the desired receive frequency in VHF versions, or 45 MHz below the desired receive frequency for UHF variants.  Make sure this LO is stable for the highest and lowest receive frequencies you require.  Again, similar comments apply to 'centering' your VCO tuning to provide adequate margin over the effects of temperature for your RX VCO.  As well, some of the comments made in the prm80mod.doc for tuning the VCOs apply - you can use a spectrum analyser which makes it easy to see how your LO tuning is progressing, or you can use a scanner receiver to 'listen' for the LO.  Generally speaking, screwing the slug 'in' or down causes the frequency of the VCO to rise (until it locks the PLL for the selected channel of course, or reaches the tuning limit for the VCO) and screwing it 'out' causes the VCO to lower it's frequency.  Bear in mind there is only so far you can go with this, and I have found that shifting a VCO 20 MHz is the most optimistic limit by standard VCO slug tuning alone.

I imagine you could open up the VCO and add or subtract some extra C or L to further modify the coverage, but the VCOs are tiny, and it is a fiddly job in the extreme to do.  If you do try this trick, the rule of thumb is to add a turn to lower the frequency coverage, and remove a turn to raise it.  I have played with a TM band unit raising it up to W1 band, and the results are pretty good, with one exception, which is noted later on.  When you remove the VCO block, be very quick with your soldering, too much heat will destroy the very fine PCB tracks.  You need to have near perfect soldering skills to do this job successfully.  Once removed, separate the two edge PCBs of the VCO from the wrap around shield.  One PCB will be marked V1 (the TX VCO): remove the waxy glue that holds the coil in place, unsolder the coil, remove about 3/4 to 1 turn of the coil, leaving just over 1 turn of wire, and replace the coil to the PCB.  Find some sort of way to re-glue the coil to the PCB - I suggest candle wax, but you can try contact glue.  This is necessary to stop VCO microphony.  The other PCB is marked V2 (the RX VCO): again remove the waxy glue, remove the coil, remove 1 turn from the coil (leaving about 2 turns) and replace the coil and re-glue the coil.  Put the PCBs back to the wrap around shield, and resolder them as you found it when removed.  Put the VCO back into the main RF PCB observing which VCO board goes where (line up V1 on the VCO with V1 on the main RF PCB), remove the slugs completely, and you should have a VCO that now covers close to 465 to 490 MHz on RX, and at least 470 to 490 MHz on TX.  When adjusting the front end receive helical filters, remove all the slugs except for helical no 3, which should have it's slug about 1/3 the way proud of the helical former.  This will tune the helicals to near the 470 to 480 MHz area.  Also spread the small free air core coil that is next to helical no 4 to improve the sensitivity.  The only problem now is the TX power amp stage itself is still filtered for TM band - producing a maximum of 2 watts at UHF CB frequencies.  This also means that the radio is pretty inefficient for the power produced at this frequency, and also heats up more than what I'd like.  But, if you can put up with that, it will save you having to find a W4 band RF assembly - these are a little harder to find than the TM band units.  I will look into modifying the filtering PCB to allow slightly more power out while still remaining effective on harmonics; this will only have a slight effect, since the RF power module is still one designed for the TM band.  The TM band PA module is a Mitsubishi M57799L (400 to 430 MHz) - if you change the PA module to M57799M (430 to 470 MHz) or M57799H (470 to 520 MHz) the power output will be much improved.  A final word of warning: you really do need to have PERFECT soldering skills to do this mod!  In any case, why bother?  Keycom has W4 band units for sale so cheap that doing this really isn't worth the effort.

There is a way to further lower the overall band covered by the VCO for the UHF versions, by replacing the metal VCO slug with a ferrite one, the coverage can be lowered further than the metal slug adjustment alone allows (but not as much as the add or remove a coil in the VCO method allows).  Because the VCO slugs are so tiny, even the smallest ferrite core or slug that you find will not fit; there are several solutions to this - either find a smaller diameter ferrite bead and drop inside the VCO tuning hole, or crush a ferrite core and deposit the bits into the tuning hole, or the most elegant solution is to grind down a ferrite core using some medium grade sandpaper.  I found that a tiny ferrite bead recovered from a Motorola handheld fitted perfectly inside my RX VCO tuning hole, and once the copper shim was replaced over the VCO, the coverage was approximately 465.5 to 490 MHz on RX from an originally W4 RF board.  The more ferrite you manage to get into the tuning hole, the lower the overall frequency band covered by the VCO.  Also, higher grade ferrite (F29 as opposed to F16 for example) will also give better results.	Conversely, to raise the coverage of a UHF PRP80 VCO, you can use a larger (i.e. longer) metallic screw to replace the existing slug, or use some brass rod or wire in place of the slug - but both these ideas have disadvantages.  Using a screw means having to find a really tiny screw, cutting off the head, and cutting a slot in the remaining threaded shaft so that you can still adjust the VCO.  Make sure you have no screw protruding above the VCO can - if you do, shorten the thread and start the tuning process again.  Using the brass rod or wire idea is OK, but is not quite as elegant or precise to tune.  To tune, simply adjust the length of the brass rod/wire that you drop in to the VCO tuning hole.  And both methods will not raise the coverage of the VCO compared to as much as ferrite cores would lower it, and either method will reduce the efficiency of the VCO - which translates to reduced VCO bandwidth covered.  I have found that using an extra slug or screw into the VCO will raise the overall coverage by no more than 5 MHz.  So, if you have a TM band unit (400 to 430 MHz) and screw the slug all the way in, you get 425 to 450 MHz, and with an extra slug or screw, you get 430 to 455 MHz.

With either method, if you end up dropping a piece of ferrite or brass etc into the tuning holes to modify the coverage, don't forget to secure the object so it can't rattle around in the hole and modify the tuning or cause microphony.  A tiny dob of hot wax will do fine, or maybe a very weak glue solution - with either of these you can remove the wax or glue if you find you at some time in the future wish to modify the VCO coverage again.

For what I imagine will be the most popular application of UHF PRP80s, the conversion of a W4 band unit to UHF CB + police RX only (in Australia, anyway) for a W4 VCO that has it’s metal slugs initially down quite deep inside the former, I have found that the W4 TX VCO only needs the slug to be screwed out to nearly the top of the VCO can, and for RX, the RX VCO slug needs to be removed, and a ferrite core or bead inserted, to get the RX working down to 465 MHz.

What I have found with tuning W4 band modules down to UHF CB and lower for 468 MHZ RX is interesting.  I’ve found that maybe 1 in 5 W4 RF modules will not go below 470 MHz, even with the ferrite in the VCO core, and maybe 1 in 8 will not go below 480 MHz at all.  Likewise, it’s maybe only 20% of W4 RF modules produced that will be able to cover right down to 465 MHz.  So, if you are doing this modification (W4 to UHF CB & 468 MHz RX) , it might be best to have two RF modules at your disposal.  You can always modify the VCO with an extra ¼ or half a turn on the VCO coils to lower the coverage, but like mentioned elsewhere, it’s a very fiddly job, you need to have perfect soldering skills.  The troublesome W4 RF modules that won’t go down to 477 / 468 MHz can be fairly simply identified – once you remove the VCO sticky covers, if you see the metal slugs near the top of the transformer, that indicates trouble.  The further down the slugs are in the former, the more likely the VCO & hence the RF module will go down to 468 MHz and below.  Note that without hex image editing, the lowest frequency FPP can program in to a modified W4 (programmed as a W1) is 465 MHz.

The next most popular application for UHF PRP80s I guess would be ham conversion to 70 cm.  This requires a TM band unit with the slugs screwed pretty much all the way down but not quite to the bottom of the former.

Once tuned to your satisfaction, don't forget to replace the adhesive foil strip over the VCO access holes and replace and resolder the copper shim over the top of that.  I actually used new copper shim, one without the perforations, so that removal is easier if it is required sometime down the road.

The TX PA stage seems to cope quite OK with transmitting lower in frequency than it was designed to, but you may experience significant power loss if you try to go up in frequency, depending on just how radical the change is.  For example, a TM band unit will do 3.5 to 4 watts at 450 MHz, but only about 1.5 to 2 watts at UHF CB etc.

Some time ago, Rockby Electronics in Australia sold some PRP80 incomplete units - pretty much a complete RF assembly with keypad & display and top panel and some other bits of hardware, but without the control boards or plastic case / shell.  I picked up one and found that it was a W4 band RF assembly, and a few minutes later had determined that it was quite happy to be retuned for UHF CB, but as mentioned previously, 468 MHz police RX only was just out of reach by the factory slugs alone.  It would seem that most of the PRP80s sold by Rockby were W4 band units, along with some TM band units, but if you did get one while Rockby were selling them, now you know how to test it, and if it is a W4 band unit, you now know how to put it to use!  Transplant it to a VHF unit and viola! a UHF CB handheld variant of the PRM8030.  I also found that the TM band units are ideal for tuning up to 450 MHz - perfect for 70cm amateur use.  I have not played with the W1 band unit, so I do not know its capabilities.  The VHF versions can certainly be tuned to cover the 2 metre band.  And before you ask, yes, Rockby have definitely sold out of the PRP80 units, quite some time ago actually, and are not able to get any more.  However, a call to Simoco (now called ComGroup - look up www.comgroup.net.au) in Melbourne reveals that the PRP80 is no longer a supported product (I get the feeling the PRP80 was considered over priced and not too popular as a result with only about 1000 units sold in Australia), and that their stock of spare RF assemblies are being sold for $150 each - they have A9 and W4 bands only.  Apparently no other spares were kept, so that is all you are likely to find.  Keycom come to the rescue with literally hundreds of RF assemblies (less the control boards or CPU) in A, T and W4 bands for $10 each, as well as hundreds of instruction manuals for the PRP80 and a few other odds and ends.  Keycom are available on their new website at:  http://keycom.d2.net.au/index.html

So, just how do you tell what band a particular RF unit is in?  Identifying a VHF unit is easy - it will have the smaller RX helical filters (see photo prp80vhf.jpg).  For UHF, however, telling a TM apart from a W1 apart from a W4 is not easy - in fact, it will be pretty much impossible, because they all look identical right down to the PCB part number.  The only sure fire way to tell is to power it up and look for where the RX local oscillator (LO) is trying to lock to.

A LO in the range of approx. 185 to 225 MHz indicates an A band unit

A LO in the range of approx. 355 to 395 MHz indicates an TM band unit

A LO in the range of approx. 420 to 455 MHz indicates an W1 band unit

A LO in the range of approx. 440 to 475 MHz indicates an W4 band unit

The most common UHF band RF assemblies produced / imported was the TM band unit, probably followed by W4 and the W1 band units.  If you have purchased a spare RF board assembly from Rockby or Philips, and it came in a cardboard box with an identifying sticker, the designations on the sticker will give a clue as to what band the unit operates in:

PRP80V8 : VHF A band (B band never imported to Australia)

PRP80U8-A : UHF TM band

PRP80U8-B : UHF U band

PRP80U8-C : UHF W1 band

PRP80U8-D : UHF W4 band

Anyway, back to the alignment info: tuning the fixed squelch setting is easy, too.  Just set your radio with the volume to a comfortable level on a non-CTCSS non voting or multiax channel, and adjust the mini pot shown in picture prp80uhf.jpg anti clockwise until the mute opens, and then slowly adjust clockwise until the mute closes.  Then continue turning the pot clockwise for another 10 or 20 degrees, otherwise you will find the mute reluctant to close after receiving a signal.

As for the four receive helical bandpass filters, they can simply be tuned by using any suitable device (metal screwdrivers are OK for the UHF versions, but non metallic must be used for VHF versions) while inserting a weak, steady & stable signal to the radio on a mid frequency channel, and using the RSSI (Received Signal Strength Indicator) as a guide.  Once again, same comments apply from the prm80mod.doc - keep the test signal weak, and you can 'simulate' a weak test signal by using either a distant repeater or using a scanner LO.  The receive front end helical filters seem to be able to adjust quite well up or down one band - for example a W4 band front end once retuned works very well for W1 frequencies, and even a bit lower.  As noted above, the TM band helicals will (only just!) make it up to the W1 band area.  Also spread or compress the small air wound coil next to helical no 4 for maximum sensitivity at your test frequencies.

I suspect that the “I.F.” adjustment as labeled on my photos are actually performing a LO mixer function, and you can try tuning that for best sensitivity if you are moving your PRP80 outside it’s design bandwidth.  UHF versions have an air wound coil near this adjustment, same comments apply – spread or compress for best sensitivity if you are moving out of the designed frequency band.	Most of the PRP80 units I have come across are 12.5 kHz versions, and unlike the PRM8030 12.5 kHz versions, the PRP80 will slightly 'clip' strongly deviated 25 kHz signals, but it is not too bad, just annoying.  If this really annoys you, I guess you could replace the ceramic 455 kHz filter, but I don't think it is worth the trouble.  To get the deviation up to normal levels, you will need to adjust the deviation pot, also shown in prp80uhf.jpg, but don't set the deviation too high - this will cause 'clipping' of your TX audio to others, and the result isn't very pretty.

As for hardware mods, that is about it - there's not too much else that you can really do.

VHF ‘ham’ 2 metre PRP80:

For VHF, I guess quite a few PRP80s will be destined for the ham 2 metre band.  The one example of this conversion I have done turned out pretty good.  There’s two ways of going about it: 1) reprogram the unit as a B band, enter the 2 metre frequencies directly and retune the front end and IF slugs for best RX performance OR 2) leave as an A9 band, and you have to use a hex editor to directly edit the divider words in your xxxxx.epr job file.  This is because the FPP band swapping technique will not allow you to quite be able to enter 2 metre frequencies.  However, it will allow you to retain the normal A9 band frequency range, and so you could also enter VHF marine, Vic CFA or country police RX freqs and so on.  An added benefit of doing things this way is that minimal tuning of the front end cores is required.  You might want to tweak the IF coil (I  suspect it’s actually a mixer coil) for better sensitivity at 2 metres, though.  Editing the hex .epr image is a real pain – and so I include here my file that I use for my 2 metre handheld – it includes all the useful FM frequencies & repeaters.  If you want to rename channels or move channels about, you can do that – just don’t try to enter or edit any of the frequency fields.  CTCSS, power, channel number or name – all fine, edit those to your hearts content, just don’t touch the frequencies!  I’ve also added the VHF marine frequencies to this – which is something I specially wanted to do, otherwise I would have just programmed the 2 metre frequencies as a B0 band – delete any unwanted channels if you want.  Look for 2metre.epr and 2metre.dsc files, and put them in a directory called 2METRE in your FPP/JOBS directory.  Something else I recommend for PRP80s is the use of one function button as ‘squelch defeat’.  The IF filters on most PRP80s were fitted for 12.5kHz channel spacing operations.  Transmissions that are slightly off frequency or over 4kHz in deviation tend to get clipped, and disabling the mute while listening to such signals helps a lot.  Programming a dedicated function button to this task makes life easier.  By the way, if you were wondering, directly editing the hex EEPROM image can manage to program frequencies from about 141 to 190 MHz for the A9 band version.  Of course, don’t think that the VCO could cover all of this, and you may have to do some retuning of the VCO if you do want to shift the coverage up or down significant amounts.  Having said that, my VHF PRP80 in A9 band covered 2 metres and selected VHF receive only channels at 165 MHz without any VCO retuning required, though if you are looking for absolute maximum sensitivity & power output efficiency at 2 metres, you would be best advised to program only 2 metres and centre the VCO tuning range for 147 MHz, as well as paying attention to the mixer adjustment.  One last point: most VHF PRP80s were used for networks around the 160 MHz area – the antenna is cut & tuned accordingly.  I would recommend obtaining an antenna specially for 147 MHz if you expect to be using the radio on that band a lot

Trunking only to PMR conversion:

(Authors note: technically, this will void the type approval of the radio, and as such may not be legal to use.  It does not affect the transmission characteristics; however proceed at your own risk…)

In the PRM8030 series of radios, the EPROM contains the firmware, which mainly determines if the radio is a PMR only, dual mode or trunking only radio, and EEPROM contains the personality.  The PRP80 departs from this slightly, using a very large EEPROM (4 megabit! an AMD AM29F040 device is used) to hold both firmware and personality.  Visual inspection shows that the control boards of the trunking only and dual mode control boards appear identical right down to the OEM part number, so the only difference must be the 'firmware' loaded into the EEPROM.  Change the 'firmware' and you in theory should be able to make the trunking only PRP80 into a dual mode type.  The 'firmware' was officially loaded at the factory by special software written through the standard programming interface.   Simoco wouldn't tell me anything more because the PRP80 range isn't a supported product any more, but I have found a way of re-flashing the firmware, email me for details if this is what you are looking to do (I promised the person who helped me find this out not to distribute it to the public).  If you want to re-flash the firmware yourself, then I guess the only avenue left open to you is the brute force way of unsoldering the device, programming it using an external writer with the appropriate firmware, and resoldering the device back in.  How do you get the firmware?  You would need to solder a few very fine wires to a 'source' EEPROM from a PMR only or dual mode unit, and using a flash reader or EPROM reader, suck the contents out onto a PC.  I have tried modifying the hardware code using FPP, hoping that FPP would write the entire firmware and personality during a write process (it takes much longer than the PRM80) but alas, it does not.  Anyone out there want to 'suck' the info out of a dual mode unit and put the image somewhere on the net for others to use?  While you are at it, it would be nice to get an archive of the PSD boot ROM code, too...

Adding a scrambler:

Adding a scrambler to the PRP80, I imagine, will be like the PRM80 - best to scramble after pre-emphasis, and de-scramble before de-emphasis.  However, due to the ultra-miniature circuitry surrounding these points, it may be more convenient to break the audio points closer to the main inter-PCB connectors J101 and J102 - these are not quite the optimum locations, but you have to take what you can get.  Yet again, all the generalised comments I made in the prm80mod.doc relating to scramblers would be equally valid here for the PRP80.

The original Shinwa handheld has a space for a large SMD IC that is custom designed for scrambling (LC8931)  - you can use this space to fit your own scrambler, however it would have to be very tiny indeed.

Turning on or off your scrambler is going to be your biggest problem.  The 'aux' function as used on the PRM80 does not work on the PRP80 - even though the display toggles a little bell symbol when 'aux' is activated (note when I say 'aux' here it does not refer to the button on the PRP80 marked "AUX" - it means a function key is programmed with the 'auxiliary' feature), there is no logic line that goes high or low in sympathy.

The original Shinwa has a logic line labeled 'crypt' which is used to turn on or off the factory scrambling IC - and the PRP80 control boards do not do anything to control this line.  Bugger.

I ended up using a flip-flop connected to the 'EMG' button on top of the radio to switch on and off my scrambler.  I also found that the best points to tap into the audio paths for scrambling are located on the back of the RF board.  This was really the scrambler install from hell!  I will never do this ever again - not unless someone offers me large sums of money, anyway!

Charging the battery

If you have purchased a PRP80 second hand, it's a fair bet that it didn't come with a battery charger.  If you look at the battery pack from the rear, you will see four metal contacts near the bottom - to charge the battery, use a standard Ni-Cd battery charger, with positive (+) going to the right most contact, and negative (-) going to the left most contact.  This applies only to the 7.5 volt 1400mAh batteries.  There was a smaller 900mAh battery that should use the same contacts.

As per usual, weak batteries or those nearing the end of their useful life can cause problems such as the radio resetting for no reason at all, problems and corruptions of memory on TX or switch on etc, so it pays to look after your batteries by cycling them properly and especially not to over charge them etc.

It is also a good idea to regularly power up your PRP80 (at least once a week I would suggest) and to keep a charged battery connected to it at all times.  This is so that the little lithium back up battery keeps the RAM maintained - a corrupt RAM due to flat batteries in theory shouldn't be a problem, but I have seen it cause a few glitches that took some effort to recover from.  If you need to replace the lithium battery on the control board, you can source these from Rockby, part no: 10971, about $4.50 each.  Also, the occasional powering up of the PRP80 while it sees little use is good 'exercise' for the PSD boot ROM.  Apparently these boot ROMs do like to get just a little regular use.  If the PSD boot ROM does fail it is a fatal problem that can not be recovered from, not without an extraordinary effort anyway.

Faults & fixes

I have only come across a couple of faults so far with the PRP80, and they are only minor problems.  If you find that your PRP80 appears to ‘forget’ the channels you had tagged for user scanning, or forgets what state it was in when you turned it off (ie RSSI on or off, trunked or PMR mode selected etc) then you have a backup battery that is dead.  It is a simple process to replace, remove the main battery and disassemble the radio, unsolder the backup button cell on the selcall board and replace, taking careful note of the polarity and being careful not to overheat the battery, board, or surrounding components.

The second fault I have observed is the PTT lever getting a bit hard to press to make the radio transmit.  This may be able to be rectified by disassembling the radio and spraying a bit of electronic grade contact cleaner into the PTT pressure contact switch.

The only other thing that might commonly go wrong is the main radio battery going flat, or just getting old with age and not being able to hold its charge any more and causing some interesting ‘low voltage’ problems.  Not really considered to be a fault, since this happens to all handheld radio batteries after a time.

Factory test mode?

Recently, I was given the job of reprogramming a PRP80, and it wasn’t going to co-operate at all.  Instead of reprogramming, it would come up with FACT TEST on the display.  I don’t know how this happened, or how a PRP80 can be forced into this mode, but if you ever do get it into this mode, here’s an explanation of what I found with it:

Once you have FACTTEST displayed, you can scroll between the following menus by using the MNU key:

SWITCHES.  Press SEL for 3 seconds to enter this mode – this will test the function buttons such as PTT, monitor, emergency (ie the five programmable function buttons)

KEYPAD.  Press SEL for 3 seconds to enter this – this will test the front keypad buttons, by displaying each button pressed on the display.

ROTARY.  Press SEL for 3 seconds to enter this – I guess this will test the top 16 position rotary switch, but I couldn’t figure that one out.

DISPLAY.  Press SEL for 3 seconds to enter this – this will test the LCD display and top panel LED.  Press 1 to light all segments, 2 to blank them all, 3 to flash all segments, 4 to invoke a cute scrolling test of the display, 5 to light the top panel LED red, 6 to light the top panel LED green, and 7 will test the backlight for the LCD display.

CHANNEL.  Press SEL for 3 seconds to enter this – then by pressing * or # you can scroll between channels 1 – 7, presumably factory test channels.

SIGNAL.  Press SEL for 3 seconds to enter this.  Then press:

1 – SATENC (CTCSS encode) – then press * or # to scroll a figure up or down.  Probably adjusts the CTCSS TX level.

2 – SATDEC (CTCSS decode) – I think this is just a test mode for CTCSS decoding.

3 – RTB (CTCSS Reverse Tone Burst) - then press * or # to scroll a figure up or down – select from none, 180 or 120 degrees.

4 – SCENC (Selcall encode) - then press * or # to scroll a figure up or down – probably adjusts the selcall tone TX level.  Selectable levels between 0-9, A, B.

5 – SCDEC (Selcall decode) – I think this just tests the decoding action of the selcall decoder.

6 – DTMF - then press * or # to scroll a figure up or down – probably sets the DTMF TX level, selectable levels of 0-9, A-F

7 – MSKENC - then press * or # to scroll a figure up or down – again, probably sets the MSK / FSK TX level, selectable levels of 0-9

8 – MSKDEC – then press PTT to get a live display of the MSK decoding action!

9 – NOMOD – I have no idea on what this is?  Possibly transmits a dead carrier for testing purposes.

0 – SIGNAL – again not sure on this one, but I think it allows reading of the RSSI figure, to check against a calibration scale?

In the above, I wasn’t sure on many of the items as I could not test them fully, the radio I was working on had a few faults – some dead keys and the RF section was definitely out of lock, so it wouldn’t receive or transmit any signals.  I’ve not been able to reproduce this factory test mode on any of my healthy PRP80s.

Oh, before you ask, this feature definitely would not be available (hidden or not) on the PRM8010, 8020, 8025 or 8030.  Something equivalent might exist for the 8040, but I doubt it after having inspected it’s text code.

What is that antenna connector?

Never seen that sort of antenna connector before?  Need to find a new antenna, but don't know what to ask for?  That connector is called a SMA female socket, and the antenna would need a SMA male plug.

OK, all this is great, but how do I program the thing?

The PRP80 has a speaker microphone socket located on the right side panel, just below the antenna, and a few of you might have guessed that it can also take an external antenna connection, so that an integrated speaker microphone and antenna can be used (like the ones you see on 'The Bill').  This connector interface is also used to program the radio personality.  The PC interface you use is almost identical to the one used for the PRM80 series - with a few minor changes.  Of course the connector going to the radio will be different, and the other major change is that in the PRM80 interface, power to run the interface comes from the PRM80.  With the PRP80, the interface must operate on an external power supply, and you only feed TX and RX data and ground lines to the PRP80.  Follow the diagram given in intfc1.jpg, but instead of wiring the input of the 78L05 to RJ45 pin 4, power this from your external power supply - 7 volts DC or more should be fine.  Don't forget to add the ground or negative connection from the power supply to the interface, too.  Where the ground connection of the interface goes to RJ45 pin 2 as shown in intfc1.jpg, take this instead to contact number 5 on the PRP80, using the below guide as a reference.  RJ45 pin 8 (MAX232 pin 11) goes to PRP80 contact 11.  RJ45 pin 7 (MAX232 pin 12) goes to PRP80 contact 8.  And finally, there needs to be 10 k ohm resistance added between contacts 11 and 12, and another 10 k ohm resistance between 12 (again) and 5.  How you manage to achieve holding all of this onto those near flat contacts will be an interesting exercise in ingenuity.  I 'cheated' and used a real PRP80 programming cable.

PRP80 interface connector field

1	2	Antenna

3	4	Connector

5	6	7	8

9	10	11	12

In summary:

Contact:		Function:

5		ground

8		TTL level data in to radio

11		TTL level data out of radio

12		External Horn alert

Exactly the same comments apply from the prm80mod.doc in relation to programming problems as they do for the PRP80 - problems are likely to be with your PC OS, so look at the suggestions for programming problems & hints in the other modification file first (PRM80mod.doc).

Typical performance specifications:

TX power: 5 watts high, 1 watt low, but both these figures are adjustable

Sensitivity: about the same as a PRM8030 : 0.28uV / -118dBm for 20dB quieting (approx 0.195uV for 12dB SINAD)

Accessories:

The PRP80 came with a good array of accessories that, while rare & hard to find, might be worthwhile keeping an eye out for:

*The programming cable is an obvious one, this is detailed in the photos below.

*A speaker microphone, with two repeated function buttons on top.

*A dual slot rapid & slow intelligent battery charger

*A single bay or four slot battery conditioner / charger.

*A car kit that allows use of the PRP80 in a vehicle.  Looking very much like the old analogue

mobile phone car kits, the PRP80 is placed in a cradle that charges the battery, allows an external

antenna to be connected, and your choice of hands free microphone operation or a standard

PRM80 fist microphone can be used, all while holding the actual radio securely in your car.

*A leather carry case with belt clip.

*A metal belt clip

Photo descriptions:

prp80uhf.jpg:	This shows a TM band UHF RF board, but all other UHF RF boards will look

identical, right down to the circuit board part number.  The main differences

will be the VCO and TX bandpass & RX helical filters.  I have also shown the

adjustment points, which will be pretty much the same for the VHF board.

prp80vhf.jpg	Here's a VHF RF board - note the smaller RX bandpass helical filters, and the lack

of a small crystal above the LR4809N IC, and an associated adjustment point.

prpctrl.jpg	A view of the two control boards that sit one on top another.  The only

adjustment on the CTCSS / DTMF / Selcall board is for the selcall tone TX level.

prpkey.jpg	Pretty boring, just a view of the keypad and display.  Note, however, the missing

screws (there's four of them) that are the only ones you need remove to allow

this display & keypad to 'hinge' away from the chassis to reveal the RF stages

below

prpfrnt.jpg	This is what the PRP80 looks like, up close & personal.

prps.jpg		Wow!  A clutch of PRP80s, with a fast charger.  Two of them are dual mode and

two are dedicated trunking.  Can't tell the difference?  Neither can I without

seeing them powered up!  The stickers on the rear of the units isn't always

helpful, either.

prpprog.jpg	This is a Philips original programming cable for the PRP80.  It plugs into a

interface box (that you can build yourself if you want) that adapts a PRM80

programming cable to allow it to program the PRP80s as well.

PRP80CK1.jpg	This is the PRP80 car kit that is very similar to a mobile phone car kit - slip the

PRP80 in, and the car kit charges the battery, connects an external antenna,

gives you the choice of fist mic or handsfree operation, and holds the PRP80

securely.

PRP80CK2.jpg	This is the electronics unit of the PRP80 car kit, showing connectors from left to

right of: Fist Mic, PTT for handsfree operation, Handsfree mic, and interface to

the cradle.

PRP80CK3.jpg	Not as clear as I would have liked, this view shows the top of the cradle, with the

charging contacts at the bottom, and the side connector that interfaces to the

PRP80.  Also, on the electronics unit is the power / speaker connector, which is

exactly the same as on the PRM80 series.

PRP80CK4.jpg	A rear view of the cradle, showing the BNC connector for the external antenna.

PRP80CK5.jpg	This is what the PRP80 looks like in the cradle.  Not a bad concept at all, really.

PRPacces.jpg	Shows the speaker microphone (looks a bit like a Kyodo KG108 speaker mike to

me), the leather protective case and the drop in charger for the PRP80.  Note how

the protective case has provision for the shorter, low capacity battery.

PRP80PI.jpg	Shows a Philips genuine programming interface for the PRP80.  Note that if you

have the PRP80 cable, and a PRM80 programming cable, the small black box is

only to 'inject' power for the interface to run, and is easily duplicated by the

hobbyist.

It's all too hard, can you help me?

You've read all this information and the information given in the prm80mod.doc, and you still think it is going to be beyond you, or you don't have the test equipment?  I may be able to help.  I can be contacted at:   mitaux8030@hotmail.com   , and I should be able to help you out, weather it be a tune up to suit your application, programming your PRP80s personality, or doing a RF assembly swap.  Give me a email to see what spares I might have that you could purchase if you want.  Please note that I do not have any complete PRP80s or control boards / CPUs at all for sale.

Other web pages to visit:

Relating to the PRP80 - Ha! You must be joking!  This is the only page I know of that has info for hobbyists about the PRP80.

Other radios in the PRP series?

Sorry, I only know about the PRP80 series.  The PRP1650 and PRP73 & PRP76 were entirely different radios, and they have n-o-t-h-i-n-g in common except that they were all sold by Philips.

Author requests:

As mentioned earlier, most of this information has been learnt from scratch, with nothing more than a good working knowledge of the PRM80 series of radios and a healthy understanding of RF principles.  Of course I will be happy to update this information if something new or interesting comes to light - just email it to me, and I will include it in this document.

Footnote from the author and important legal disclaimers:

The information presented here is to be used by you entirely at your own risk.  I have not verified all modifications listed, I simply don't have the time or resources.  However, I have verified maybe 95% of the mods, and I am reasonably confident that the remainder should all work as described.  Nor will I be responsible for how you use this information, or any consequences resulting from its use.  Some (well, most, actually!) of these modifications will void the type approval of the radio for all but amateur use.

This information was put together with the intention that all may benefit from it - the information is made available to you free of charge, and that is the way I want it to remain.  Many hundreds of dollars & hours of effort have gone in to these projects to experiment (into a dummy load, of course) and learn to produce this document.  I wouldn't be too pleased to learn that someone has used this information for blatant profiteering.  By all means, help others with their hobby, but do it for a reasonable fee.

Most important of all, if you do learn of an error in my information or find a better way of doing things, or have some more or additional information to suggest, please share it with us! E-mail me at: mitaux8030@hotmail.com  and I will update this file so that we all may benefit.

I would also welcome suggestions as to what you would like to see added to this file to make it more useful.  Any reasonable suggestion will be looked at.

Version history:

1.0	First write up.

1.1	Some more info on firmware changing, and VCO tuning.

1.2	Photos added

1.3	Minor corrections.

1.4	Re-flashing the firmware updated, and TM to W1 band conversion discussed.

1.5	A few new bits of information

1.6	The programming interface is simpler than I thought.

1.7	Scrambler info added.

1.8	Deleted reference to the service manual.  No service manual was ever produced for the PRP80.

1.9	Added info of Keycoms PRP80 stock.  Well done Brad!

2.0	Rockby have UHF antennae available cheap.

2.1	Added info on keeping your PRP80 healthy.

2.2	A little info on the RF PA modules used in the UHF versions of the PRP80.

2.3	Corrected spelling.

2.4	Rockby have the lithium backup battery available if you find yours is unable to maintain the RAM.

2.5	Updated the sensitivity measurement.

2.6	Added an accessories section and photos of the PRP80 car kit.

2.7	Corrected spelling

2.8	One minor correction.

2.9	Added photos of the leather protective case, belt clip and speaker microphone.

3.0	Added a new photo of a genuine Philips PRP80 programming lead setup.

3.1	The only fault I have ever found with a PRP80, and how to fix it.

3.2	Another fault added, and I have discovered U0 band PRP80s do exist in Aussie.

3.3	Rockby have run out of handheld antennae

3.4	Found a built in factory test mode, but it will have to remain a bit of a mystery.

3.5	Added info on programming for 2 metres, and tidied up a few sections, especially in relation to IF

adjustment.

  - Important update on programming interface.

  - More info on W4 tuning down to UHF CB & below.

- Legal disclaimers added, and no more belt clips available.